- To fill fresh water tank:

  - Connect fresh water supply and open valve located exterior of trailer.

  - Open tank fill valve in the equipment room.  When water starts flowing out of the overflow, located under the trailer, close the tank fill valve.

- To operate using the fresh water tank system:

  - Open tank bypass valve.

  - Open pump access valve.

  - Close the crossover valve.

  - Close antifreeze valve.

  - Make sure breaker is turned on for pump to work.

- All shower supply valves must be left open. Shower supply valves are located behind showers in either the equipment room or in exterior access panels.

- On-Demand Hot Water Heater:

  - Make sure breakers are turned on located in breaker box.

  - Make sure water heater is plugged in and breaker is turned on.  Note:  The unit is shipped with (2) 40lb. LP tanks that must be filled prior to use.

c.   Check LP connections and gas supply.

d.   Open gas valve (located in equipment room).

Note for LP tank change:  Do not bypass the LP pressure regulator.  Always connect any LP tank to the main regulator at the front of the trailer.  Bypassing this regulator will cause the water heater(s) to fail and void the warranty.

To Discharge Grey Water System

- Make sure fresh water pump is on and valves are in default setting.

- Make sure filtered water discharge line is capped off (Located exterior of trailer).

- Open pump prime valve.  Allow all three canisters to fill with water then close valve.

- To operate discharge system:

  - Open filtered water discharge valve.

  - Make sure breaker is turned on for waste pump.

  - Make sure waste pump is plugged in and switch is turned on located on pump.

  - Depress pump discharge switch until pump stays running by itself.

- Discharge pump will run until waste tank is empty, then automatically shut off.

- If waste pump does not stay on, repeat process.

Revised 7/28/26